# Arduino CLI Installation & TinZr Auto-Flashing Guide (Windows)

## 1. Download Arduino CLI

Download the Windows 64‑bit Arduino CLI from the official installation page:

https://arduino.github.io/arduino-cli/1.3/installation/

## 2. Install Arduino CLI

1. Download the Windows ZIP package.

2. Extract it.

3. Create a folder (recommended):

C:\Users\<YOUR_USERNAME>\Tools\arduino-cli\

4. Move arduino-cli.exe into that folder.

## 3. Test the Installation

Open a new Command Prompt and run:

C:\Users\<YOUR_USERNAME>\Tools\arduino-cli\arduino-cli.exe version

## 4. Install ESP32 Support

Run these commands once:

C:\Users\<YOUR_USERNAME>\Tools\arduino-cli\arduino-cli.exe core update-index

C:\Users\<YOUR_USERNAME>\Tools\arduino-cli\arduino-cli.exe core install esp32:esp32

## 5. Configure a.py

Set the Arduino CLI path inside your Python script:

ARDUINO_CLI = r"C:\Users\<YOUR_USERNAME>\Tools\arduino-cli\arduino-cli.exe"

Set the sketch path:

SKETCH_PATH = r"C:\path\to\ActivateOTA\ActivateOTA.ino"

Set board type (FQBN):

FQBN = "esp32:esp32:esp32c3"

## 6. Running the Auto-Flashing Tool

Run the tool:

python a.py

What it does:

- Scans COM ports

- Detects TinZr hardware (VID 303A)

- Uploads the same .ino sketch to all detected TinZr boards

## Done!

Your TinZr boards are updated successfully.